ACTIVITY 5

ESP8266 & DHT11 Humidity Temperature Monitor on ThingSpeak

Aim: Upload humidity & temperature data to Thing Speak, periodically logging ambient light level to Thing  Speak

Procedure

In this experiment, we will learn how to monitor humidity and temperature using the DHT11 sensor and  NodeMCU ESP8266, and then upload the readings to the ThingSpeak cloud server. The DHT11 will be used  to measure temperature and humidity, while ThingSpeak will serve as our IoT cloud platform to store, visualize,  and analyze the sensor data from anywhere in the world.

This guide is based on the Arduino IDE and is specifically written for the NodeMCU ESP8266 board.  However, the same concept can be applied to any ESP8266-based development board for real-time temperature  and humidity monitoring.

Components Required

• NodeMCU ESP8266 Board

• DHT11 Sensor

• Breadboard

• Jumper Wires

Software Required

• Things Speak Account is needed (We have to create cloud server account).

• Circuit Diagram and Connections

To interface the DHT11 temperature and humidity sensor with the NodeMCU ESP8266, we only need three  active pins: VCC, Data, and GND. The DHT11 communicates using a single-wire digital interface, which makes  the wiring very simple.

In the circuit shown above, the DATA OUT pin of the DHT11 is connected to GPIO0 (D3) of the NodeMCU,  and the sensor is powered through the NodeMCU’s 3.3V supply. Since the ESP8266 GPIO pins work at 3.3V  logic, the DHT11 is fully compatible and does not require any level shifting.

Setting Thingspeak & Getting API Key:

To store and visualize your sensor data on the cloud, we will be using ThingSpeak, a popular IoT analytics  platform. Follow the steps below to create your channel and get the required API Key.

1. Create a ThingSpeak Account

Visit https://thingspeak.com/ and sign in. If you are a new user, create a free account by completing  a simple registration process. Once your account is activated, log in to your dashboard.

2. Create a New Channel

After login, click on “New Channel” from the dashboard.

• Enter a Channel Name (e.g., Humidity & Temperature Monitoring)

• Enable at least two fields:

• Field 1 → Temperature

• Field 2 → Humidity

• Add an optional description and metadata if needed

Scroll down and click “Save Channel” to create it successfully.

3. Get the API Key

Once your channel is created, navigate to the “API Keys” tab.

Here you will find two keys:

• Write API Key – used for uploading data from NodeMCU

• Read API Key – used for reading/displaying data

Copy the Write API Key and save it in a notepad — you will need this key later in the  Arduino code to send your DHT11 data to ThingSpeak.

Source Code/Program:

The complete program for monitoring Humidity and Temperature using the DHT11 sensor and  NodeMCU on ThingSpeak is provided below. Follow the steps to upload it through the Arduino IDE:

1. Copy and paste the code into your Arduino IDE.

2. Download and install the DHT11/DHT22 library from GitHub (or through Arduino  Library Manager) and ensure it is added to your project.

3. Select the correct board by navigating to:

Tools → Board → NodeMCU 1.0 (ESP-12E Module).

4. Insert your ThingSpeak Write API Key into the code at the specified location. This is the  same key you generated while creating your channel.

5. Edit the Wi-Fi credentials, replacing the default SSID and Password with your own network  details.

6. Verify (Compile) the code and then upload it to the NodeMCU board.

/*

NodeMCU (ESP8266) + DHT11 → ThingSpeak

--------------------------------------

- Reads temperature & humidity from DHT11

- Posts data to ThingSpeak Channel fields

- Uses a safe 20 s upload interval (ThingSpeak min = 15 s; DHT11 ~2 s)

Wiring (NodeMCU 1.0 ESP-12E):

DHT11 VCC -> 3V3

DHT11 DATA -> D3 (GPIO0) // you can change the pin if you prefer  DHT11 GND -> GND

(Most DHT11 modules already have a 10k pull-up on DATA)

*/

#include <ESP8266WiFi.h>

#include <DHT.h>

// -------------------- USER SETTINGS --------------------

const char* ssid = "how2electronics"; // <-- replace with your WiFi SSID const char* password = "alhabibi"; // <-- replace with your WiFi password

// ThingSpeak settings

const char* server = "api.thingspeak.com";

const char* writeAPI = "H38TEGNC0XKW43BB"; // <-- replace with your ThingSpeak WRITE  API key

// Map your fields in the channel: Field1 = Temperature (°C), Field2 = Humidity (%RH)

// Sensor settings

#define DHTPIN 0 // GPIO0 (NodeMCU pin label: D3)

#define DHTTYPE DHT11

// ------------------------------------------------------

DHT dht(DHTPIN, DHTTYPE);

WiFiClient client;

// Upload every ≥15 s. Use 20 s for safety.

const unsigned long UPLOAD_INTERVAL_MS = 20000;

unsigned long lastUpload = 0;

void connectWiFi() {

Serial.print("Connecting to WiFi: ");

Serial.println(ssid);

WiFi.mode(WIFI_STA);

WiFi.begin(ssid, password);

// Wait for connection

uint8_t tries = 0;

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print('.');

if (++tries >= 60) { // ~30 seconds timeout

Serial.println("\nWiFi connect timeout. Restarting...");

ESP.restart();

}

}

Serial.println("\nWiFi connected!");

Serial.print("IP address: ");

Serial.println(WiFi.localIP());

}

void setup() {

Serial.begin(115200);

delay(50);

Serial.println();

Serial.println("DHT11 + NodeMCU → ThingSpeak");

dht.begin();

connectWiFi();

}

bool postToThingSpeak(float temperatureC, float humidity) {

// Build URL-encoded POST body. API key can go in header (X-THINGSPEAKAPIKEY)  // or as field "api_key" in the body. We’ll send it in the header and keep  // the body to fields only.

String postBody = "field1=" + String(temperatureC, 1) + "&field2=" + String(humidity, 1);  if (!client.connect(server, 80)) {

Serial.println("Connection to ThingSpeak failed.");

return false;

}

// HTTP request

client.print(String("POST /update HTTP/1.1\r\n") +

"Host: " + String(server) + "\r\n" +

"Connection: close\r\n" +

"X-THINGSPEAKAPIKEY: " + String(writeAPI) + "\r\n" +  "Content-Type: application/x-www-form-urlencoded\r\n" +  "Content-Length: " + String(postBody.length()) + "\r\n\r\n" +  postBody + "\r\n");

// Read response (optional but useful for debugging)

unsigned long start = millis();

while (client.connected() && millis() - start < 3000) {

while (client.available()) {

String line = client.readStringUntil('\n');

// Uncomment to see entire response:

// Serial.println(line);

// ThingSpeak returns an integer line with the entry number if successful  if (line.length() == 1 && line[0] == '\r') {

// blank line before body

String body = client.readString();

Serial.print("ThingSpeak response (entry ID or 0): ");  Serial.println(body);

break;

}

}

}

client.stop();

return true;

}

void loop() {

// Respect ThingSpeak & DHT11 timing

if (millis() - lastUpload < UPLOAD_INTERVAL_MS) {  delay(50);

return;

}

// Read sensor

float h = dht.readHumidity();

float t = dht.readTemperature(); // Celsius

if (isnan(h) || isnan(t)) {

Serial.println("Failed to read from DHT11. Retrying next cycle...");

lastUpload = millis(); // still wait until next interval

return;

}

Serial.print("Temperature: ");

Serial.print(t, 1);

Serial.print(" °C, Humidity: ");

Serial.print(h, 1);

Serial.println(" %RH");

// Upload to ThingSpeak

if (postToThingSpeak(t, h)) {

Serial.println("Update sent to ThingSpeak.");

} else {

Serial.println("Update failed.");

}

lastUpload = millis();

}

Once the upload is complete and your NodeMCU is powered on, it will automatically connect to Wi-Fi and begin  sending DHT11 sensor data to your ThingSpeak channel.

Monitor Humidity Temperature Data on ThingSpeak

Once powered, the DHT11 continuously measures humidity and temperature from the surrounding air. The  NodeMCU requests the sensor data through the DATA pin, and then sends the collected values to  the ThingSpeak cloud via Wi-Fi.

To monitor your sensor readings on ThingSpeak, open your channel and navigate to the “Private View” section.  The temperature and humidity data will be displayed as real-time graphs, updating each time your NodeMCU  uploads new values.

Watch a video on YouTube. https://www.youtube.com/watch?v=jYjuxWUefhg